If you need Shorty just press and leave the button. Program will take it as shorty. If you need Short, press button until you hear beep. if you release button now it will be short. If you keep on pressing you will hear second beep and the program will register this as Long press.

PWM driving led has time period of 2ms.

PWM driving motor has time period of 1ms.

Wake Up Stage:

Initially when the system is power on and button is press the program wakes up in wake up stage. If you keep on pressing the button until the first beep program will register it as Short and move to Stage A Slow.For shorty program remain in wake up stage with counter for wake up stage reset. This stage last for 5 sec during this after every 1 sec battery status is checked and displayed.

Table 1:Button Action in Wake Up Stage

| Shorty | Reset Stage counter (Wake up stage will now last for 5 more seconds) |
| --- | --- |
| Short | Move to Stage A Slow |
| Long | Beep but does nothing |

Shutdown Stage:

In shut down stage program stops motor and led pwm, produce three short beeps of 50 ms and clear POWER_TAKEOVER so that the system is off.

Stage A Slow:

In Stage A Slow, the motor will start running at 30% pwm for 1 minute. If the long button is pressed during this program will shift to stage shutdown if short is pressed it moves to stage StageBNormal. For now, Shorty in this stage move to Stage Doff.  Green led blinks during stage A with on duration of 500ms after every 1 sec.

Table 2: Button Action in Stage A Slow

| Shorty | Move to Stage Doff |
| --- | --- |
| Short | Move to Stage B |
| Long | Move to Shutdown Stage |

Stage B Normal:

In Stage B, the motor will start running at 100%. Leds will now show state of battery. If the long button is pressed during this program will shift to stage shutdown if short is pressed it moves to stage StageBNormal. Shorty in this stage move to Stage Doff.

Table 2: Button Action in Stage A Slow

| Shorty | Move to Stage Doff |
| --- | --- |
| Short | Move to Stage C |
| Long | Move to Shutdown Stage |

StageCEnd:

In stageCEnd, on entering this stage motor runs at 50% duty cycle. Battery status is checked and displayed for two seconds. After that blue led is turn on. The stage last for 2 minutes if no button is pressed. After that program moves to Stage Shutdown.

Table 2: Button Action in Stage CEnd

| Shorty | Move to Stage Doff |
| --- | --- |
| Short | Move to Shutdown Stage |
| Long | Move to Shutdown Stage |

StageDOff:

In stage Doff Motor is off, there is single beep after every 5 sec if button press is long it moves to Shutdown. Battery status is also indicated for 1 sec after every beep.

Table 2: Button Action in Stage D Off

| Shorty | Indicate battery status for 1 sec |
| --- | --- |
| Short | Move to last Stage |
| Long | Move to Shutdown Stage |

Check Battery:

checkBattery function is used to check the battery status for now if voltage is below 3.1 set by value of NOGO it is Kill_Alert. No matter what stage the checkBattery was called if it is Kill Alert program will move to shut down.

If voltage is above 3.1V and less than 3.3V set by Value of REDWARN it is REDWARN indicated by red led . Also give double beep.

If Voltage is above 3.3V and below 3.6V set by value of YELWARN it is YELLOW_ALERT indicated by yellow led.

For voltage above 3.6V it is Normal Green indicated by green led.